CISCO NETWORKING PROJECT

ENTERPRISE NETWORK

DHCP & PORT SECURITY

Cisco Packet Tracer Implementation

PROJECT TYPE

Networking/Network Engineering

TECHNOLOGIES IMPLEMENTED

IPv4 Addressing • VLANs • 802.1Q Trunking
Router-on-a-Stick • Inter-VLAN Routing
DHCP • Port Security • MAC Address Security
Network Verification • Troubleshooting

PREPARED BY
Anwar Ul Haq

TOOL USED
 Cisco Packet Tracer

PROJECT
Project 02

DATE
October 2026

TABLE OF CONTENT

1. PROJECT OVERVIEW	5

2. PROJECT OBJECTIVES	6

3. NETWORK SCENARIO	7

Department and VLAN Structure	7

Network Architecture	7

4. NETWORK TOPOLOGY	8

5. IP ADDRESSING & VLAN DESIGN	9

5.1 VLAN Design	9

5.2 IP Addressing Plan	9

5.3 VLAN-to-Port Assignment	9

6. CONFIGURATION	11

6.1 VLAN Configuration	11

6.2 Access Port Configuration	11

6.3 Trunk Configuration	11

6.4 Router-on-a-Stick Configuration	11

6.5 DHCP Configuration	12

6.6 Port Security Configuration	12

6.7 Configuration Summary	13

7. VERIFICATION & TESTING	14

7.1 VLAN Verification	14

7.2 Trunk Verification	15

7.3 Router Interface Verification	16

7.4 DHCP Pool Verificattion	16

7.5 DHCP Binding Verification	17

7.6 PC DHCP Configuration Verification	18

7.7 Same-VLAN Connectivity Test	18

7.8 Inter-VLAN Connectivity Test	19

7.9 Port Security Verification	21

7.10 Port Security Violation Test	22

7.11 Port Security Recovery	22

7.12 Router Configuration Verification	23

7.13 Switch Configuration Verification	26

8. TROUBLESHOOTING CASE STUDY	28

8.1 Problem Introduction	28

8.2 Security Violation Condition	28

8.3 Connectivity Failure	28

8.4 Configuration Correction and Recovery	29

8.5 Verification After Correction	29

8.6 Root Cause Analysis	29

9. RESULTS	30

10. FUTURE IMPROVEMENTS	31

LIST OF FIGURES

Figure 1   Enterprise Network Topology	8

Figure 2 VLAN Configuration Verification	14

Figure 3 Trunk Link Verification	15

Figure 4 Router Interface Verification	16

Figure 5 DHCP POOL VERIFICATION	17

Figure 6 DHCP Binding Verification	17

Figure 7 PC DHCP Configuration Verification	18

Figure 8 Same Vlan Connectivity Test	19

Figure 9 Inter-VLAN Connectivity Test — IT	20

Figure 10. Inter-VLAN Connectivity Test — Finance	21

Figure 11  Port Security Configuration Verification	21

Figure 12 Port Security Violation Test	22

Figure 13 Port Security Recovery	23

Figure 14 Router Running Configuration — Part 1	24

Figure 15 Router Running Configuration — Part 2	25

Figure 16 Switch Running Configuration — Part 1	26

Figure 17 Switch Running Configuration — Part 2	27

# 1. PROJECT OVERVIEW

This project presents the design and implementation of a small enterprise network using Cisco Packet Tracer. The network consists of three departments: Human Resources (HR), Information Technology (IT), and Finance.

The network was designed using VLAN segmentation to logically separate departmental traffic. Router-on-a-Stick was implemented to provide communication between the VLANs through a single router interface using IEEE 802.1Q trunking.

Building on the VLAN and inter-VLAN routing concepts from Project 01, this project introduces two important enterprise networking concepts: Dynamic Host Configuration Protocol (DHCP) and switch port security.

DHCP was configured on the Cisco router to automatically provide IP addresses, subnet masks, default gateways, and DNS server information to the client PCs. Port security was configured on the access ports to restrict the number of allowed MAC addresses and prevent unauthorized devices from accessing the network.

The project also includes connectivity testing, DHCP verification, port-security violation testing, recovery of a secured port, and configuration verification using Cisco IOS commands.

# 2. PROJECT OBJECTIVES

The main objectives of this project are:

- Design and implement a small enterprise network using Cisco Packet Tracer.

- Segment the network into separate departmental VLANs for HR, IT, and Finance.

- Configure IEEE 802.1Q trunking between the switch and router.

- Implement Router-on-a-Stick for inter-VLAN communication.

- Configure the Cisco router as a DHCP server.

- Automatically assign IP addresses and network parameters to client PCs.

- Configure switch port security to control authorized devices.

- Implement static and sticky MAC address security.

- Simulate an unauthorized-device connection and observe the security violation.

- Recover the secured port and restore authorized network access.

- Verify network operation using Cisco IOS commands and connectivity tests.

- Document the configuration, verification, troubleshooting, and results.

# 3. NETWORK SCENARIO

The network is designed for a small enterprise organization consisting of three departments: Human Resources (HR), Information Technology (IT), and Finance. Each department contains two end devices and is assigned to a separate VLAN to provide logical network segmentation.

A Cisco 2960 Layer 2 switch is used to connect the end devices, while a Cisco 2911 router provides Layer 3 gateway functionality and enables communication between the different VLANs. The switch and router are connected through a single trunk link configured using IEEE 802.1Q encapsulation.

### Department and VLAN Structure

| Department | VLAN | Network Address | Default Gateway | Devices |
| --- | --- | --- | --- | --- |
| Human Resources (HR) | 10 | 192.168.10.0/24 | 192.168.10.1 | PC1, PC2 |
| Information Technology (IT) | 20 | 192.168.20.0/24 | 192.168.20.1 | PC3, PC4 |
| Finance | 30 | 192.168.30.0/24 | 192.168.30.1 | PC5, PC6 |

### Network Architecture

The network follows a centralized design in which the Cisco switch provides connectivity for all six end devices. The router is connected to the switch using a trunk link and uses separate subinterfaces for VLAN 10, VLAN 20, and VLAN 30.

Each VLAN represents a separate broadcast domain. Devices within the same VLAN can communicate directly through the switch, while communication between different VLANs is handled by the router through Router-on-a-Stick configuration.

The design provides a simple and scalable foundation for an enterprise network while demonstrating essential concepts including VLAN segmentation, access ports, trunking, router subinterfaces, and inter-VLAN routing.

# 4. NETWORK TOPOLOGY

The network topology was implemented in Cisco Packet Tracer using one Cisco 2911 router, one Cisco 2960 switch, and six end devices. The six PCs are divided into three departments: Human Resources (HR), Information Technology (IT), and Finance.

The switch provides Layer 2 connectivity for all end devices. The connection between the switch and router is configured as an IEEE 802.1Q trunk, allowing traffic from VLAN 10, VLAN 20, and VLAN 30 to traverse a single physical link.

The router uses Router-on-a-Stick configuration, with separate subinterfaces assigned to each VLAN. These subinterfaces provide the default gateways required for communication between different VLANs.

Figure 1 shows the complete network topology implemented for this project.

Figure 1   Enterprise Network Topology

# 5. IP ADDRESSING & VLAN DESIGN

VLAN segmentation was implemented to logically separate the three departments within the enterprise network. Each department was assigned a unique VLAN and IPv4 subnet. DHCP was configured on the Cisco 2911 router to automatically assign IP addresses and network parameters to the end devices.

## 5.1 VLAN Design

Three VLANs were created on the Cisco 2960 switch. VLAN 10 was assigned to the Human Resources department, VLAN 20 to the Information Technology department, and VLAN 30 to the Finance department.

| VLAN ID | VLAN Name | Department | Network |
| --- | --- | --- | --- |
| 10 | HR | Human Resources | 192.168.10.0/24 |
| 20 | IT | Information Technology | 192.168.20.0/24 |
| 30 | FINANCE | Finance | 192.168.30.0/24 |

## 5.2 IP Addressing Plan

P addresses were assigned automatically to the end devices using DHCP. The router subinterface for each VLAN was configured as the default gateway for devices within that VLAN.

| Device | Department | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| PC1 | HR | 10 | 192.168.10.13 | 255.255.255.0 | 192.168.10.1 |
| PC2 | HR | 10 | 192.168.10.12 | 255.255.255.0 | 192.168.10.1 |
| PC3 | IT | 20 | 192.168.20.11 | 255.255.255.0 | 192.168.20.1 |
| PC4 | IT | 20 | 192.168.20.12 | 255.255.255.0 | 192.168.20.1 |
| PC5 | Finance | 30 | 192.168.30.12 | 255.255.255.0 | 192.168.30.1 |
| PC6 | Finance | 30 | 192.168.30.11 | 255.255.255.0 | 192.168.30.1 |

## 5.3 VLAN-to-Port Assignment

The switch access ports were assigned according to the department structure. Each department's two PCs are connected to access ports belonging to the same VLAN.

| Switch Port | Connected Device | VLAN | Mode |
| --- | --- | --- | --- |
| Fa0/2 | PC1 | 10 | Access |
| Fa0/3 | PC2 | 10 | Access |
| Fa0/4 | PC3 | 20 | Access |
| Fa0/5 | PC4 | 20 | Access |
| Fa0/6 | PC5 | 30 | Access |
| Fa0/7 | PC6 | 30 | Access |
| Fa0/1 | R1 G0/0 | 10, 20, 30 | Trunk |

# 6. CONFIGURATION

The network devices were configured using the Cisco IOS command-line interface in Cisco Packet Tracer. The configuration process involved creating the required VLANs, assigning switch access ports, configuring the trunk connection, implementing Router-on-a-Stick, configuring DHCP services, and enabling switch port security.

## 6.1 VLAN Configuration

Three VLANs were created on the Cisco 2960 switch to provide logical separation between the three departments.

vlan 10

name HR

vlan 20

name IT

vlan 30

name FINANCE

## 6.2 Access Port Configuration

Three VLANs were created on the Cisco 2960 switch to provide logical separation between the three departments.

vlan 10

name HR

vlan 20

name IT

vlan 30

name FINANCE

## 6.3 Trunk Configuration

The connection between SW1 Fa0/1 and R1 G0/0 was configured as an IEEE 802.1Q trunk. This allows traffic from multiple VLANs to travel through the same physical link.

interface fa0/1

switchport mode trunk

## 6.4 Router-on-a-Stick Configuration

Router subinterfaces were configured on R1 to provide default gateways for the three VLANs and enable inter-VLAN routing.

interface g0/0

no shutdown

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface g0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

## 6.5 DHCP Configuration

The Cisco 2911 router was configured as a DHCP server to automatically assign IP addresses, default gateways, and DNS information to the end devices.

ip dhcp excluded-address 192.168.10.1 192.168.10.10

ip dhcp excluded-address 192.168.20.1 192.168.20.10

ip dhcp excluded-address 192.168.30.1 192.168.30.10

ip dhcp pool HR

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

ip dhcp pool IT

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 8.8.8.8

ip dhcp pool Finance

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 8.8.8.8

## 6.6 Port Security Configuration

Port Security was configured on the switch access ports to limit each port to one authorized MAC address and protect the network from unauthorized devices.

Fa0/2 was configured with a manually assigned static MAC address, while Fa0/3 to Fa0/7 were configured to learn MAC addresses using sticky learning.

interface fa0/2

switchport mode access

switchport port-security

switchport port-security mac-address 00d0.d344.6cb1

switchport port-security maximum 1

switchport port-security violation shutdown

interface fa0/3

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

interface range fa0/4-7

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security violation shutdown

switchport port-security mac-address sticky

## 6.7 Configuration Summary

The completed configuration establishes three separate VLANs, assigns the appropriate end devices to each VLAN, carries VLAN traffic through the trunk link, and provides inter-VLAN routing through Router-on-a-Stick. DHCP automatically assigns network parameters to the end devices, while Port Security restricts access to authorized MAC addresses.

The complete router and switch configurations are also provided separately in the project Configuration folder.

# 7. VERIFICATION & TESTING

After completing the network configuration, verification commands and connectivity tests were performed in Cisco Packet Tracer. The purpose of verification was to confirm that the VLANs, trunk link, router subinterfaces, DHCP services, port security, and inter-VLAN communication were operating correctly.

## 7.1 VLAN Verification

The show vlan brief command was used on SW1 to verify that VLAN 10, VLAN 20, and VLAN 30 were successfully created and that the appropriate switch ports were assigned to each VLAN.

Figure 2. VLAN Configuration Verification

Figure 2 VLAN Configuration Verification

The verification output confirms that the required VLANs are present and that the end-device ports are assigned to their corresponding VLANs.

## 7.2 Trunk Verification

The show interfaces trunk command was used to verify the trunk connection between SW1 Fa0/1 and R1 G0/0.

Figure 3. Trunk Link Verification

Figure 3 Trunk Link Verification

The output confirms that Fa0/1 is operating in trunking mode using IEEE 802.1Q encapsulation and that VLANs 10, 20, and 30 are active on the trunk.

.

## 7.3 Router Interface Verification

The show ip interface brief command was used on R1 to verify the operational status of the router's physical interface and VLAN subinterfaces. The verification confirms that the configured interfaces are operational and available for inter-VLAN routing.

Figure 4 Router Interface Verification

Figure 4 Router Interface Verification

The output shows that the router subinterfaces for VLAN 10, VLAN 20, and VLAN 30 are configured with the correct gateway addresses and are operational. The output shows that the router subinterfaces for VLAN 10, VLAN 20, and VLAN 30 are configured with the correct gateway addresses and are operational.

## 7.4 DHCP Pool Verificattion

The show ip dhcp pool command was used on R1 to verify the DHCP pools configured for the HR, IT, and Finance departments. The verification confirms that each pool contains the correct network address and that DHCP is actively providing addresses to the corresponding VLANs.

Figure 5 DHCP POOL VERIFICATION:

Figure 5 DHCP POOL VERIFICATION

## 7.5 DHCP Binding Verification

The show ip dhcp binding command was used on R1 to verify the IP addresses dynamically assigned to the connected PCs. The output displays the leased IP addresses and corresponding MAC addresses of the DHCP clients.

Figure 6 DHCP Binding Verification

Figure 6 DHCP Binding Verification

The verification output confirms that DHCP successfully assigned IP addresses to all six end devices across VLAN 10, VLAN 20, and VLAN 30.

## 7.6 PC DHCP Configuration Verification

The ipconfig command was used on PC1 to verify the network configuration received from the DHCP server. The output was checked to confirm the assigned IPv4 address, subnet mask, and default gateway.

Figure 7 PC DHCP Configuration Verification

Figure 7 PC DHCP Configuration Verification

The output confirms that PC1 successfully received its IPv4 configuration through DHCP, including the IP address 192.168.10.13, subnet mask 255.255.255.0, and default gateway 192.168.10.1.

## 7.7 Same-VLAN Connectivity Test

A connectivity test was performed between PC1 and PC2, which are both members of VLAN 10 (HR). The test verifies communication between devices within the same VLAN after obtaining their IP addresses through DHCP.

Figure 8. Same-VLAN Connectivity Test

Figure 8 Same Vlan Connectivity Test

The successful ping responses confirm that PC1 and PC2 can communicate within VLAN 10.

## 7.8 Inter-VLAN Connectivity Test

Inter-VLAN connectivity was tested by sending ICMP ping traffic between devices belonging to different VLANs. PC1 in VLAN 10 was tested against PC3 in VLAN 20 and PC5 in VLAN 30.

Figure 9 Inter-VLAN Connectivity Test — IT

Figure 9 Inter-VLAN Connectivity Test — IT

The successful ping responses confirm that PC1 in VLAN 10 can communicate with PC3 in VLAN 20 through the Router-on-a-Stick configuration.

Figure 10. Inter-VLAN Connectivity Test — Finance

Figure 10. Inter-VLAN Connectivity Test — Finance

The successful ping responses confirm that PC1 in VLAN 10 can communicate with PC5 in VLAN 30 through the Router-on-a-Stick configuration.

## 7.9 Port Security Verification

Port Security was configured on the switch access ports to restrict each port to one authorized MAC address. The show port-security command was used on SW1 to verify the Port Security status and configuration of the protected access ports.

Figure 11  Port Security Configuration Verification

Figure 11  Port Security Configuration Verification

The verification output confirms that Port Security is enabled on Fa0/2 through Fa0/7, with a maximum of one secure MAC address configured for each port. The violation mode is set to Shutdown, and no security violations were recorded during normal operation.

## 7.10 Port Security Violation Test

An unauthorized-device test was performed to verify the operation of Port Security. PC2 was temporarily connected to switch port Fa0/2, which was configured with the authorized MAC address of PC1.

A ping was attempted from the unauthorized device, causing a MAC address security violation. The switch detected the unauthorized MAC address and placed Fa0/2 into the Secure-shutdown state according to the configured violation mode.

Figure 12 Port Security Violation Test

Figure 12 Port Security Violation Test

The verification output shows that the port entered Secure-shutdown status and recorded a security violation. The last source address displayed the unauthorized MAC address detected on the port.

## 7.11 Port Security Recovery

After the security violation, the authorized PC1 was reconnected to Fa0/2. The secured port was manually recovered by shutting down and reactivating the interface.

The port was then verified using the show port-security interface fa0/2 command to confirm that the port returned to the Secure-up state and that the authorized MAC address was active.

Figure 13 Port Security Recovery

Figure 13 Port Security Recovery

The verification output confirms that Fa0/2 returned to the Secure-up state with the authorized MAC address configured and no active security violation.

## 7.12 Router Configuration Verification

The show running-config command was used on R1 to review the complete active router configuration. The configuration was verified to confirm the DHCP exclusions, DHCP pools, Router-on-a-Stick subinterfaces, VLAN encapsulation, and gateway addresses.

Figure 14 Router Running Configuration — Part 1

Figure 14 Router Running Configuration — Part 1

Figure 15 Router Running Configuration — Part 2

Figure 15 Router Running Configuration — Part 2

The configuration output confirms that the router contains the required DHCP services and Router-on-a-Stick configuration for VLAN 10, VLAN 20, and VLAN 30.

## 7.13 Switch Configuration Verification

The show running-config command was used on SW1 to review the complete active switch configuration. The configuration was verified to confirm the VLANs, access port assignments, 802.1Q trunk configuration, and Port Security settings.

Figure 16 Switch Running Configuration — Part 1

Figure 16 Switch Running Configuration — Part 1

Figure 17 Switch Running Configuration — Part 2

Figure 17 Switch Running Configuration — Part 2

The configuration output confirms that the switch contains the required VLAN configuration, access port assignments, trunk configuration, and Port Security settings for the enterprise network.

# 8. TROUBLESHOOTING CASE STUDY

## 8.1 Problem Introduction

During the testing phase, a Port Security violation was intentionally simulated to verify the network's ability to detect and block an unauthorized device. Fa0/2 on SW1 was configured with the authorized MAC address of PC1 and a maximum limit of one MAC address.

PC2 was then temporarily connected to Fa0/2. Because PC2 had a different MAC address, the switch detected a security violation and placed the interface into the Secure-shutdown state. This demonstrated the effectiveness of Port Security in preventing unauthorized devices from accessing the network.

## 8.2 Security Violation Condition

Before the test, Fa0/2 was configured to allow only the authorized MAC address of PC1. The port was limited to one secure MAC address, and the violation mode was set to shutdown.

The relevant Port Security configuration was:

interface fa0/2

switchport mode access

switchport port-security

switchport port-security mac-address 00d0.d344.6cb1

switchport port-security maximum 1

switchport port-security violation shutdown

The configuration was intentionally left unchanged while PC2 was connected to the port. Since PC2 had a different MAC address, its connection triggered the Port Security violation.

## 8.3 Connectivity Failure

PC2 was temporarily connected to Fa0/2, which was configured with the authorized MAC address of PC1. A connectivity test was performed by attempting to ping the router gateway at 192.168.10.1.

The connection was blocked because the MAC address of PC2 did not match the secure MAC address configured on Fa0/2. The switch detected the violation and automatically placed the interface into the Secure-shutdown state.

Figure 12 Port Security Violation Test

The verification output showed the unauthorized MAC address as the last source address and recorded a security violation on Fa0/2. This confirmed that Port Security successfully detected and blocked the unauthorized device.

## 8.4 Configuration Correction and Recovery

To restore authorized network access, PC2 was disconnected from Fa0/2 and the authorized PC1 was reconnected to the port. The interface was then reset using the following commands:

interface fa0/2

shutdown

no shutdown

After the interface was reset, PC1 was able to access the network through Fa0/2. The show port-security interface fa0/2 command was then used to verify that the port had returned to the Secure-up state.

Figure 13 Port Security Recovery

The verification output confirmed that Fa0/2 was operational, had one configured secure MAC address, and recorded no active security violation after recovery.

## 8.5 Verification After Correction

After the port was recovered, the authorized PC1 was tested again to confirm normal network operation. The show port-security interface fa0/2 command verified that the interface was in the Secure-up state and that the authorized MAC address was present.

The security violation count returned to zero after recovery, confirming that the unauthorized device was no longer connected and that the port was operating normally.

Connectivity from PC1 was restored successfully, demonstrating that the Port Security violation had been resolved without affecting the authorized device.

## 8.6 Root Cause Analysis

The root cause of the connectivity failure was a mismatch between the MAC address authorized on Fa0/2 and the MAC address of the connected device.

Fa0/2 was configured to allow only one specific MAC address belonging to PC1. When PC2 was connected, its different MAC address exceeded the security policy configured on the port. The switch therefore detected a Port Security violation and placed the interface into the Secure-shutdown state.

The issue was resolved by removing the unauthorized device, reconnecting the authorized PC1, and resetting the interface. This troubleshooting process demonstrated how Port Security can detect unauthorized access and how a secured interface can be recovered after a violation.

# 9. RESULTS

The enterprise network was successfully implemented and tested in Cisco Packet Tracer. The following results were achieved:

- Successfully created and configured VLANs for HR, IT, and Finance.

- Successfully assigned access ports and configured IEEE 802.1Q trunking.

- Successfully implemented Router-on-a-Stick for inter-VLAN communication.

- Successfully configured DHCP on the router and automatically assigned IP addresses and network parameters to all six PCs.

- Successfully verified DHCP pools, bindings, and client configurations.

- Successfully implemented Port Security with static and sticky MAC address configurations.

- Successfully detected and blocked an unauthorized device through the Port Security violation test.

- Successfully recovered the secured interface and restored authorized network connectivity.

- Successfully verified same-VLAN and inter-VLAN connectivity.

- Successfully verified the final router and switch configurations using Cisco IOS commands.

Overall, the project demonstrated the practical implementation of VLAN segmentation, inter-VLAN routing, DHCP, MAC address security, network verification, and troubleshooting in a small enterprise network environment.

# 10. FUTURE IMPROVEMENTS

The current project provides a foundation for further development into a larger and more secure enterprise network. The following improvements can be implemented in future projects:

- Implement OSPF for dynamic routing between multiple routers.

- Configure standard and extended ACLs to control network traffic.

- Implement NAT and PAT for external network connectivity.

- Configure SSH for secure remote management of network devices.

- Implement STP and EtherChannel for Layer 2 redundancy and improved network availability.

- Introduce firewall, VPN, and IDS/IPS concepts for advanced network security.

- Use Linux and Python for network automation and device management.

- Expand the topology to include multiple switches, routers, and additional departments.

- Explore cloud networking concepts and integrate enterprise networking with cloud infrastructure.

These improvements will build upon the networking concepts demonstrated in this project and provide a pathway toward advanced Network Engineering, Network Security, Automation, and Cloud Networking.